Programme Principal:

/********************************************************************/

/********************************************************************/

/**				 The ROBOT of GEII-Warrior			 **/

/**				      The Master Game				 **/

/**											 **/

/**					XXXX-XXX-XXXX				 **/

/********************************************************************/

/********************************************************************/

/*Programme*/

/********************************************************************/

/* erreur au niveau des capteurs: gauche et droit non identique.	  */

/* moteur à 100% 									  */

/* IN 1&2 à utiliser pour régler les capteurs 	 			  */

/* vitesse=25%, delta_vfort=20, delta_vdoux=15  				  */

/* vitesse=50%, delta_vfort=35, delta_vdoux=25		    		  */

/* vitesse=100%, delta_vfort=80, delta_vdoux=60				  */

/*											  */

/* par convention, tout ceux qui touche au niveau des capteurs, c'est*/ /* par une vue de arriére au robot.						  */

/* Sens de marche.!!!								  */

/*	pour arthur: gauche == babord /droit == tribord			  */

/* moteur gauche = 0								  */

/* moteur droit = 1									  */

/********************************************************************/

#include <pic.h>

#include "télémétre.h"

#include "can.h"

#include "pwm.h"

#include "direction.h"

#include "démarrage.h"

#include "arrivée.h"

#include "delay.h"

#define JACK RD0

void jack(void);

unsigned char capt1;

unsigned char i;

void main(void)

{

TRISD1=0;	TRISB2=0;TRISB1=0;TRISB5=0;TRISB4=1;

initpwm();		/******************************************/

InitCan();		/*   	initialisation des fonctions 		 */

init_telem();		/*							 */

micro_switch();	/*							 */

RECUL=0; 		/******************************************/

jack();			/*							 */

démarrage();		/******************************************/

capt1=0;

while(1)

{

for(i=0;i<5;i++)

{

capt1 = (capt1 << 1) | seuil(convcapt(i));

/* on récupère les valeurs des capteurs de*/

/* gauche à droite, sens de course		 */

capt1=0x1f & capt1;

}

/* ici on détermine la position du robot par*/

/* rapport a la ligne				  */

direction (capt1);

/* on décide de la direction et on déduit  */

/* les valeurs de la pwm 			  */

if(RB4==0)

{

arrivée();	/* pour l'arrivée, détection par contact de*/

/* type interrupteur (Tout ou Rien)		  */

}

}

}

/********************************************************************/

/*				 							  */

/* cette fonction permet de faire démarrer le robot environ une  	  */

/* secondes après/*avoir enlevé le jack. Elle force la position sur */

/* stop en attendant la consigne de départ.				  */

/*										 	  */

/********************************************************************/

void jack(void)

{

while(JACK==0)

{

stop(); 		/* on attend la consigne de départ   */

DJAUNE=1;DROUGE=1;DVERT=1;

}

for(i=0;i<5;i++)

{

DelayMs(50);     /* on attend 5 fois 50ms			  */

}

}

Fonctions :

_Démarrage.c :

#include <pic.h>

#include <stdlib.h>

#include "pwm.h"

#include "can.h"

void démarrage(void)

{

unsigned char i,capt=0;		/* départ avant la ligne */

while(capt==0)		/* tant qu'on ne trouve pas de */

/* ligne on avance tout droit  */

{

for(i=0;i<5;i++)

{

capt = (capt << 1) | seuil(convcapt(i));

capt=0x1f & capt;

}

setdutycycle( 1 , 100); /* moteur à fond		 */

setdutycycle( 0 , 100); /* moteur à fond		 */

}

}

Démarrage.h :

#ifndef _DEMARRAGE_H

#define _DEMARRAGE_H

void démarrage(void);

#endif

_Arrivée.c :

#include <pic.h>

#include <stdlib.h>

#include "pwm.h"

#include "direction.h"

#include "delay.h"

void arrivée(void)

{

char i;

RECUL=1;

setdutycycle(1,25);

setdutycycle(0,25);

for(i=0;i<3;i++)

{

DelayMs(25);

}

DROUGE=0;

DVERT=1;

while(1)

{

RECUL=0;			/* lorsqu’on est arrive, */

setdutycycle(1,0);	/* le robot stop et fait */

setdutycycle(0,0);	/* une marche arriére	 */

for(i=0;i<6;i++)

{

DelayMs(25);

}

DROUGE=!DROUGE;

DVERT=!DVERT;

}

}

Arrivée.h :

#ifndef _ARRIVEE_H

#define _ARRIVEE_H

void arrivee(void);

void DelayMs(unsigned char);

#endif

_Direction.c :

/***********************************************************/

/*										*/

/*	DIRECTION , douxdroite, fortdroite , douxgauche ,	*/

/*    fortgauche , droit , recul , raccourci , stop , 	*/

/*	croisement								*/

/*										*/

/*  ces fonctions sont utilisées pour les commandes moteur	*/

/*										*/

/*										*/

/***********************************************************/

#include <pic.h>

#include <stdlib.h>

#include <stdio.h>

#include <math.h>

#include "pwm.h"

#include "direction.h"

#include "télémetre.h"

#include "can.h"

#include "delay.h"

int 		dist,	intcroi,	intrac,	inttele,	intrec;

unsigned int 	perdu=0,	rac=0	;

unsigned char 	vitesse=100,delta_vfort=82,delta_vdoux=70;

/* le fort est terriblement bien, vue les grosses courbures*/

/* le doux est enclenché pendant  un temps trop court:	*/

/* solution :								*/

/* faire une boucle pour un doux se décrémentant petit à	*/

/* petit mais le laisser suffisamment de temps			*/

/*   initialisation de la vitesse (max 100%)			*/

/*   initialisation du rapport des virages			*/

/*   il faut que le % soit plus faible en fort			*/

/***********************************************************/

/* on doit faire varier la valeur à envoyer sur le moteur	*/

/* 2 cas considerés comme impossibles 101 , 000			*/

/* on a num=1 quand on est sur la ligne

/* et on a num=0 quand on est sur la moquette (normalement)*/

/* attention les diodes sont actives sur le 0 	(négatif)	*/

/***********************************************************/

void direction (unsigned char cap)

{

if (cap!=0)

{

RECUL=0; perdu=0;

}

switch(cap)

{

case 0x01: fortgauche(); 	DJAUNE=1;DROUGE=0;DVERT=0;		break;

case 0x02: douxgauche(); 	DJAUNE=1;DROUGE=1;DVERT=0;	break;

case 0x03: fortgauche(); 	DJAUNE=1;DROUGE=0;DVERT=0;	break;

case 0x04: toutdroit(); 	DJAUNE=0;DROUGE=1;DVERT=0;	break;

case 0x06: douxgauche(); 	DJAUNE=1;DROUGE=1;DVERT=0;	break;

case 0x08: douxdroite(); 	DJAUNE=0;DROUGE=1;DVERT=1;	break;

case 0x10: fortdroite(); 	DJAUNE=0;DROUGE=0;DVERT=1;	break;

case 0x0c: douxdroite(); 	DJAUNE=0;DROUGE=1;DVERT=1;	break;

case 0x18: fortdroite(); 	DJAUNE=0;DROUGE=0;DVERT=1;	break;

case 0x00: recul(); 		DJAUNE=1;DROUGE=0;DVERT=1;	break;

default : stop();		DROUGE=1;DVERT=1;DJAUNE=1;		break;

}

}

void douxgauche(void)

{

setdutycycle( 0 , vitesse-delta_vdoux);

setdutycycle( 1 , vitesse);

}

void fortgauche(void)

{

setdutycycle( 0 , (vitesse-delta_vfort));

setdutycycle( 1 , vitesse);

}

void douxdroite(void)

{

setdutycycle( 1 , (vitesse-delta_vdoux));

setdutycycle( 0 , vitesse);

}

void fortdroite(void)

{

setdutycycle( 1 , (vitesse-delta_vfort));

setdutycycle( 0 , vitesse);

}

void toutdroit(void)

{

setdutycycle( 1 , vitesse);

setdutycycle( 0 , vitesse);

}

void recul (void)

{

if(intrec==0)

{

perdu++;

if(perdu>1000)

{

RECUL=1;

setdutycycle(1 ,50 );

setdutycycle(0 ,50 );

}

if(perdu>3000)

{

perdu=0;

RECUL=0;

}

}

}

void stop(void)

{

setdutycycle( 1 , 0);

setdutycycle( 0 , 0);

}

void ralenti(void)

{

setdutycycle( 1 , 75);

setdutycycle( 0 , 75);

}

Direction.h :

#ifndef _REDIRECT_H

#define _REDIRECT_H

#define RECUL RD1			/* pour la marche arrière	*/

#define DROUGE RB2

#define DVERT RB1

#define DJAUNE RB5

void douxdroite(void);

void fortdroite(void);

void douxgauche(void);

void fortgauche(void);

void toutdroit(void);

void recul (void);

void stop(void);

void ralenti(void);

void direction (unsigned char cap);

#endif

_PWM.c :

/***********************************************************/

/*										*/

/*				PWM ET INIT_PWM				*/

/*										*/

/*ces fonctions sont utilisées pour les commandes moteur	*/

/*										*/

/*										*/

/***********************************************************/

#include <pic.h>

#include <stdlib.h>

/***********************************************************/

/* initialisation CCP1 et CCP2 en mode PWM avec une		*/

/* frequence de 15khz et un quartz de 20 MHz,on a calculer */

/* PR2= 100 % du rapport cyclique du signal de 15khz       */

/***********************************************************/

void initpwm(void)

{

PR2= 82;

CCPR1L=0;

CCP1CON=0x0c;  	/* on met a 0 les bits 4 et 5 pour */

/* se positionner en mode PWM		*/

TRISC2=0;      	/*			mise en sortie	*/

TRISC1=0;      	/*			mise en sortie	*/

CCPR2L=0;

CCP2CON=0x0c;

T2CON=0x05;    	/*	les bits 0 et 1 de t2con sont */

/* 	le prescaleur			*/

}

/***********************************************************/

/*                                       				*/

/*				setdutycycle				*/

/*										*/

/* cette fonction permet de mettre un signal ayant un 	*/

/* rapport cyclique choisi sur le moteur choisi.		*/

/* le numéro du moteur et le rapport cyclique sont entres	*/

/* en paramètre.   					            */

/* si le numéro du moteur ou le rapport présente une erreur*/

/* alors les sorties ccp1 et ccp2 sont mises a 0.          */

/***********************************************************/

void setdutycycle(char num , unsigned char dutycycle )

{

char val1=PR2;								/*RECUL=0;*/  /*pour avancer*/

unsigned int dut;

if ((dutycycle > 100))

{

CCPR1L =0;

CCPR2L =0;

return;

}

dut= ( (unsigned int) val1 * (unsigned int) dutycycle ) / 100;			/* on transpose en % */

dut = dutycycle;

if ( num==0)					/*moteur 1*/

{

CCPR1L=dutycycle;

return;

}

if ( num==1)					/*moteur 2*/

{

CCPR2L=dutycycle;

return;

}

CCPR1L =0;

CCPR2L =0;

return;

}

PWM.h :

#ifndef _PWM_H

#define _PWM_H

void initpwm(void);

void setdutycycle(char  , unsigned char  );

#endif

_Can.c :

/***********************************************************/

/*										*/

/*				CONVCAPT , INITCAN ET waitRAT		*/

/*										*/

/*		ces fonctions sont utilisées pour le CAN		*/

/*										*/

/*										*/

/***********************************************************/

#include <pic.h>

#include <stdlib.h>

void waitRAT(void);

unsigned char counter;

void InitCan(void)

{

ADCON1 = 0x00 ; 		/* Port A&E analogique ,*/

/* justification gauche */

ADCON0 = 0x81 ; /* Tad = 32*Tosc = 32 * 50ns = 	 */

/* 1.6µs >= 1.6µs(cf p116), ADON ;  */

ADIF = 0 ;      	/* interdit les interruptions */

}

unsigned char convcapt(unsigned char ch)

{

ADCON0 = ADCON0 & 0xC7 ;

/* RAZ CHSx Sélection du canal */

if(ch==4)

{

CHS2=1;

}

if((ch==3) || (ch==2))

{

CHS1=1;

}

if((ch==1) || (ch==3))

{

CHS0=1;

}

waitRAT();         /* Attendre le temps d'acquisition */

ADGO = 1 ;	/* Démarre la conversion 		  */

/* Attend la fin de conversion 	  */

while (ADGO && ADON) ;

/* Par sécurité on teste aussi ADON  */

/* car sinon on rentre dans une boucle infinie */

return ADRESH;

}

/***********************************************************/

/* Required acquisition time for A/D module form Max.	*/

/* Heise 2002 p. 115 equation 11-1 Tacq=19,72,e-6 seconds, */

/* approx. 2,e-5 seconds. One instruction is executed in 	*/

/* one cycle, one cycle at 20MHz takes 200ns. Thus we must */

/* idle for 100 instructions. Formula for this loop		*/

/* Inst(I)=1+1+( (I-1)*(1+2) )+2+2=6+3I-3=3I+3+2 		*/

/* Inst.Cycles for the call of this function =>Inst(I)=3I+5*/

/* Inst(I) must be 400 100=3I+5 => I=29				*/

/* Timing critical, so this is still in assembler.		*/

/***********************************************************/

void waitRAT(void)

{

#asm

I EQU 54; 	/*	nous avons doublé la valeur de I	*/

/*	pour plus de sécurité		*/

movlw  I; 		/*	1 top d’horloge machine		*/

movwf  _counter; 	/*	1 top d’horloge machine		*/

WaitRAT_Loop

decfsz _counter,f  ; 	/* 1 top d horloge machine 	*/

/* si I!=0, autrement deux 	*/

/* tops d horloge			*/

goto   WaitRAT_Loop ; 	/* 2 tops d horloge		*/

#endasm

}

/***********************************************************/

/*										*/

/* cette fonction permet de déterminer si le robot se 	*/

/* trouve sur la piste, en fonction de la variable des 	*/

/* capteurs.								*/

/*										*/

/***********************************************************/

unsigned char seuil(unsigned char v )

{

int num;

if((0x01 <= v)  && (v < 0xA0) )

{

num=1;

}

else

{

if((0x90 <=v) && (v< 0xFE))

{

num=0;

}

else num = 0x0f;

}

return num;

}

Can.h :

#ifndef _CAN_H

#define _CAN_H

void InitCan(void);

unsigned char convcapt(unsigned char ch);

unsigned char seuil(unsigned char v );

#endif

_I2C.c :

#include	<pic.h>

#include	"delay.h"

#include 	"i2c.h"

/***********************************************************/

/*	I2C functions for HI-TECH PIC C - master mode only 	*/

/*										*/

/* TIMING -see Philips Sem. 80C51-based 8-bit ucontrollers,*/

/* 	         or e.g. Philips PCF8574 data sheet		*/

/* 	Send stop condition - data low-high while clock high	*/

/***********************************************************/

i2c_Stop(void)

{  		/* don't assume SCL is high on entry	*/

SCL_HIGH();

SDA_LOW();               /* ensure data is low first */

DelayUs(I2C_TM_DATA_SU);

SCL_DIR = I2C_INPUT;		/* float clock high 	*/

DelayUs(I2C_TM_STOP_SU);

SDA_HIGH();        /* the low->high data transistion */

DelayUs(I2C_TM_BUS_FREE);	/* bus free time before */

/* next start 		*/

SDA_DIR = I2C_INPUT;		/* float data high 	*/

return;

}

/***********************************************************/

/* 	Send (re)start condition - ensure data is high then 	*/

/* issue a start condition - see also i2c_Start() macro	*/

/***********************************************************/

i2c_Restart(void)

{

SCL_LOW();                    /* ensure clock is low */

SDA_HIGH();                   /* ensure data is high */

DelayUs(I2C_TM_DATA_SU);

SCL_DIR = I2C_INPUT;		 /* clock pulse high */

DelayUs(I2C_TM_SCL_HIGH);

SDA_LOW();               /* the high->low transition */

DelayUs(I2C_TM_START_HD);

return;

}

/***********************************************************/

/* 	Send a byte to the slave - returns true on error	*/

/***********************************************************/

unsigned char

i2c_SendByte(unsigned char byte)

{

signed char i;

for(i=7; i>=0; i--)

{

SCL_LOW();                   /* drive clock low 	*/

/* data hold time = 0, send data now 	*/

SDA_DIR = ((byte>>i)&0x01);

if ((byte>>i)&0x01)            /* bit to send 	*/

{

SDA_HIGH();

}

else

{

SDA_LOW();

}

DelayUs(I2C_TM_DATA_SU);

SCL_DIR = I2C_INPUT;		/* float clock high 	*/

if(i2c_WaitForSCL())	   /* wait for clock release	*/

return TRUE;			/* bus error 	*/

DelayUs(I2C_TM_SCL_HIGH);	/* clock high time 	*/

}

return FALSE;

}

/***********************************************************/

/* 	send an address and data direction to the slave		*/

/* 	  - 7-bit address (lsb ignored)				*/

/* 	  - direction (FALSE = write )				*/

/***********************************************************/

unsigned char

i2c_SendAddress(unsigned char address, unsigned char rw)

{

return i2c_SendByte(address | (rw?1:0));

}

/***********************************************************/

/* 	Check for an acknowledge					*/

/* 	  - returns ack or ~ack, or ERROR if a bus error	*/

/***********************************************************/

signed char

i2c_ReadAcknowledge(void)

{

unsigned char ack;

SCL_LOW();                      /* make clock is low */

SDA_DIR = I2C_INPUT;		 /* disable data line   */

/* -listen for ack 	*/

DelayUs(I2C_TM_SCL_TO_DATA);		/* SCL low to data*/

/* out valid 	*/

SCL_DIR = I2C_INPUT;		/* float clock high 	*/

DelayUs(I2C_TM_DATA_SU);

ack = SDA;				/* read the acknowledge */

/* wait for slave to release clock  */

/* line after processing byte 	*/

if(i2c_WaitForSCL())

return I2C_ERROR;

return ack;

}

/***********************************************************/

/* 	Read a byte from the slave					*/

/* 	  - returns the byte, or I2C_ERROR if a bus error	*/

/***********************************************************/

int

i2c_ReadByte(void)

{

unsigned char i;

unsigned char byte = 0;

for(i=0; i<8; i++)

{

SCL_LOW();                      /* drive clock low */

DelayUs(I2C_TM_SCL_LOW);	/* min clock low  period */

SDA_DIR = I2C_INPUT;		/* release data line */

SCL_DIR = I2C_INPUT;		/* float clock high */

if(i2c_WaitForSCL())

return I2C_ERROR;

DelayUs(I2C_TM_SCL_HIGH);

byte = byte << 1;		/* read the next bit */

byte |= SDA;

}

return (int)byte;

}

/***********************************************************/

/* 	Send an (~)acknowledge to the slave

/* 	  - status of I2C_LAST implies this is the last byte */

/*	to be sent								*/

/***********************************************************/

void

i2c_SendAcknowledge(unsigned char status)

{

SCL_LOW();

if ( status & 0x01) {

SDA_LOW();       /* drive line low -> more to come */

}

else

{

SDA_HIGH();

}

DelayUs(I2C_TM_DATA_SU);

SCL_DIR = I2C_INPUT;			/* float clock high */

DelayUs(I2C_TM_SCL_HIGH);

return;

}

/***********************************************************/

/* Send a byte to the slave and acknowledges the transfer 	*/

/* 	  - returns I2C_ERROR, ack or ~ack				*/

/***********************************************************/

signed char

i2c_PutByte(unsigned char data)

{

if(i2c_SendByte(data))

return I2C_ERROR;

return i2c_ReadAcknowledge();	/* returns ack, ~ack */

}

/***********************************************************/

/* Get a byte from the slave and acknowledges the transfer	*/

/* 	  - returns true on I2C_ERROR or byte			*/

/***********************************************************/

int

i2c_GetByte(unsigned char more)

{

int byte;

if((byte = i2c_ReadByte()) == I2C_ERROR)

return I2C_ERROR;

i2c_SendAcknowledge(more);

return byte;

}

/***********************************************************/

/* Send an array of bytes to the slave and acknowledges the*/

/* transfer									*/

/*  - returns number of bytes not successfully transmitted */

/***********************************************************/

int

i2c_PutString(const unsigned char *str, unsigned char length)

{

signed char error;

while(length)

{

if((error = i2c_PutByte(*str)) == I2C_ERROR)

return -(int)length;		/* bus error */

else

if(error)

return (int)length;	/* non acknowledge */

str++;

length--;

}

return FALSE;					/* everything OK */

}

/*

* 	Reads number bytes from the slave, stores them at str and acknowledges the transfer

* 	  - returns number of bytes not successfully read in

*/

unsigned char

i2c_GetString(unsigned char *str, unsigned char number)

{

int byte;

while(number)

{

if((byte = i2c_GetByte(number-1)) == I2C_ERROR)

return number;			/* bus error */

else

*str = (unsigned char)byte;

str++;

number--;

}

return FALSE;				/* everything OK */

}

/***********************************************************/

/* 	Opens communication with a device at address. Mode	*/

/* 	indicates I2C_READ or I2C_WRITE.				*/

/* 	  - returns TRUE if address is not acknowledged		*/

/***********************************************************/

unsigned char

i2c_Open(unsigned char address, unsigned char mode)

{

i2c_Start();

i2c_SendAddress(address, mode);

if(i2c_ReadAcknowledge())

return TRUE;

return FALSE;

}

/***********************************************************/

/* 	wait for the clock line to be released by slow slaves*/

/* 	  - returns TRUE if SCL was not released after the	*/

/* 	    time out period.						*/

/* 	  - returns FALSE if and when SCL released		*/

/***********************************************************/

unsigned char

i2c_WaitForSCL(void)

{   			/* SCL_DIR should be input here */

if(!SCL)

{

DelayUs(I2C_TM_SCL_TMO);

/* if the clock is still low -> bus error */

if(!SCL)

return TRUE;

}

return FALSE;

}

unsigned char i2c_read(unsigned char ucAdr)

{

unsigned char ucDat;

if (i2c_ReadFrom(ucAdr)==0)

{

ucDat=i2c_GetByte(I2C_MORE);

i2c_Stop();

}

return(ucDat);

}

I2C.h :

#ifndef	_I2C_H_

#define _I2C_H_

/***********************************************************/

/*	SDA (data) and SCL (clock) bits				*/

/*										*/

/*				Special note!!!				*/

/*										*/

/*	If the clock and data lines are in the same port, you*/

/*	will need to beware of the Read/Modify/Write issue in*/

/*	the PIC – since a bit set or clear on any one bit in */

/*	a port will read and write back all other bits, any 	*/

/*	bits configured as input which 				*/

/***********************************************************/

/* Uncomment the next line to use the PIC's SSP Module*/

#define I2C_MODULE 1

#ifdef I2C_MODULE			/* I2C module uses PORT C */

#define SCL             RC3   /* clock on port C bit 2 */

#define SCL_DIR         TRISC3

#define SDA     RC4            /* data on port C bit 1 */

#define SDA_DIR         TRISC4

#define I2CTRIS TRISC

#define MASTER_MODE     0B1011 /* I2C firmware controlled */

/* Master Mode (slave idle) */

#define SSPMode(val)   SSPCON &=0xF0; SSPCON|=(val & 0xf)

#else		/* Change port as required - defaults to port b */

#define	SCL		RB2		/* clock on port B bit 2 */

#define	SCL_DIR		TRISB2

#define SDA             RB1         /* data on port B bit 1 */

#define	SDA_DIR		TRISB1

#define I2CTRIS TRISB

#endif

#define M_SDA_INP	0x02

#define M_SDA_OUT   0xFD

#define M_SCL_INP   0x04

#define M_SCL_OUT	0xFB

#define I2C_INPUT	1		/* data direction input */

#define I2C_OUTPUT	0	/* data direction output */

#define I2C_READ	0x01		/* read bit used with address */

#define I2C_WRITE	0x00		/* write bit used with address */

#define FALSE		0

#define TRUE		!FALSE

#define I2C_ERROR	(-1)

#define I2C_LAST	FALSE	/* SendAck: no more bytes to send */

#define I2C_MORE	TRUE		/* SendAck: more bytes to send */

#define i2c_Start()		i2c_Restart()

#define i2c_WriteTo(address)	i2c_Open((address), I2C_WRITE)

#define i2c_ReadFrom(address)	i2c_Open((address), I2C_READ)

#ifdef I2C_MODULE

#define SCL_HIGH() SCL_DIR = I2C_INPUT

#define SCL_LOW()  SCL_DIR = I2C_OUTPUT

#define SDA_HIGH() SDA_DIR = I2C_INPUT

#define SDA_LOW()  SDA_DIR = I2C_OUTPUT

#else

#define SCL_HIGH() SCL = 1; SCL_DIR = I2C_OUTPUT

#define SCL_LOW()  SCL = 0; SCL_DIR = I2C_OUTPUT

#define SDA_HIGH() SDA = 1; SDA_DIR = I2C_OUTPUT

#define SDA_LOW()  SDA = 0; SDA_DIR = I2C_OUTPUT

#endif

/***********************************************************/

/* Timings for the i2c bus. Times are rounded up to the 	*/

/* nearest micro second.						*/

/***********************************************************/

#define I2C_TM_BUS_FREE		5

#define	I2C_TM_START_SU		5

#define I2C_TM_START_HD		4

#define I2C_TM_SCL_LOW		5

#define	I2C_TM_SCL_HIGH		4

#define I2C_TM_DATA_SU		1

#define I2C_TM_DATA_HD          0

#define I2C_TM_SCL_TO_DATA	4	/* SCL low to data valid */

#define	I2C_TM_STOP_SU		4

#define I2C_TM_SCL_TMO		10	/* clock time out */

extern signed char	i2c_ReadAcknowledge(void);

extern unsigned char	i2c_SendAddress(unsigned char, unsigned char);

extern unsigned char	i2c_SendByte(unsigned char);

extern int		i2c_ReadByte(void);

extern void		i2c_Restart(void);

extern void		i2c_Stop(void);

extern void		i2c_SendAcknowledge(unsigned char);

extern signed char	i2c_PutByte(unsigned char);

extern int		i2c_GetByte(unsigned char);

extern unsigned char	i2c_Open(unsigned char, unsigned char);

extern unsigned char	i2c_GetString(unsigned char *, unsigned char);

extern int		i2c_PutString(const unsigned char *, unsigned char);

extern unsigned char	i2c_WaitForSCL(void);

extern void 		i2c_Free(void);

#endif			/* _I2C_H_ */

_Telemétre.c :

/********************************************************/

/* Programme de test lcd+i2c+sonar sur 16F877	         */

/* A compiler avec lcd.c  delay.c i2c.c	    	         */

/* Ce programme affiche la distance mesurée par le sonar*/

/* la température lue par le DS1621 et la luminosité    */

/* ambiante vue par le module sonar sur l'écran lcd     */

/* le lcd est en mode 4 bits(seuls D4-D7 sont utilisees)*/

/* Voir le câblage dans lcd.c                           */

/* Auteur : Arlotto	Date : 03/05/02   création       */

/* 	   Arlotto	       12/02/03 	       		   */

/********************************************************/

#include <pic.h>

#include <stdlib.h>

#include "delay.h"

#include "i2c.h"

void WriteSonar(unsigned char location, unsigned char byte);

unsigned char ReadSonar(unsigned char location);

#define DS1621   0x48<<1 /* adresse i2c du ds1621 */

#define SONAR    0xE0    /* adresse i2c du module sonar */

void init_telem(void)

{

TRISC3=0;

/*************** initialize i2c module *****************/

#ifdef I2C_MODULE

SSPMode(MASTER_MODE);

SSPEN = 1;

CKP = 1;

#else

#error "I2C NON INITIALISE"

#endif

}

int telem(void)

{

unsigned char light,distH,distL,Cpt;

int dist;

/* Sonar */

/*Lecture luminosité pour s'assurer que le sonar répond */

/* (pour l'instant on recommence tant que ça ne marche pas!) */

WriteSonar(0,0x51);

DelayMs(25);

Cpt=0;

do {

light=ReadSonar(0);

Cpt++;

}while(light==255 && Cpt<255);

/* Lecture Distance en cm */

DelayMs(25);

distH=ReadSonar(2);

DelayMs(25);

distL=ReadSonar(3);

dist = (distH<<8) + distL ;

DelayMs(25);

return dist;

}

/* ecrit un octet sur le sonar */

void WriteSonar(unsigned char location, unsigned char byte)

{

i2c_WriteTo(SONAR);

i2c_PutByte(location);

i2c_PutByte(byte);

}

/* lit un octet sur le sonar */

unsigned char ReadSonar(unsigned char location)

{

i2c_WriteTo(SONAR);

i2c_PutByte(location);

i2c_ReadFrom(SONAR);

return i2c_GetByte(I2C_LAST);

}

Telemétre.h :

#ifndef	_telem_H_

#define _telem_H_

void init_telem(void);

int telem(void);

#endif